ANSWER TO QUIZ ON SPEED FRAME WITH REASONING

Q.1 Top cone drum gives drive to bottom cone drum

Ans  B- True

Reason- ( Top cone drum is the driver, it gives motion to the bottom cone drum by means of a belt. Top cone drum gets drive from motor via a train of gears)

Q.2 To avoid roller slip in 4/4 roller drafting system

Ans D- We decrease the draft between 2nd & 3rd roller

Reason- ( Roller slip occurs at second top roller. So to avoid roller slip we have to reduce the speed of the 2nd top roller. Now the surface speed of a roller is directly proportional to draft, hence the draft in between the 2nd & 3rd roller is reduced. Therefore the draft between these two rollers is just about 1, i.e. tension draft. This is one of the suggestions of Shirley Institute which I had discussed in class.)

Q.3 If there was a problem of undrafted ends in a ring frame, what parameter would you consider changing in a  roving frame?

Ans - B

Reason-  ( Undrafted means the fibres in the rove did not slide over each other. This means that the fibres are held more tightly than desired, and if you recollect in speed frame we impart twist in rove so as to impart strength. So here we have to consider the amount of twist given in speed frame.)

Q.4 Typical roving count is

Ans - A

Reason - (Draw frame sliver count is 0.1 To 0.16 Ne and draft in speed frame is around +/-  10)

Q.5 The speed of bobbin rail in roving frame

Ans - B

Reason- ( As the bobbin builds up, the length of each coil increases, so now more time will be required to lay each coil. The length of coil increases because  bobbin dia increases and length of a coil = ᴨ x dia. So if the rate of movement of bobbin rail does not decrease as the bobbin builds up, then the coils that will laid on the bobbin will not be next to each other, but there will be a gap between successive coils)

Q.6 Flyer lead type of winding is preferable for winding of cotton rovings.

Ans - B

Reason - ( For cotton and man made fibre and its blends we always use bobbin leading speed frames. In bobbin leading the tension on the rove is less and if it breaks the broken end stays on the bobbin surface)

Q.7 Flyer speed in roving frame

Ans - C

Reason - ( Flyer is mounted on a spindle and the spindle drives the flyer. The spindle rpm is always the same i.e. it does not change)

Q.8 Typical draft given in roving frame is

Ans - A ( please refer to ppt)

Q.9. When the number of teeth in the DCP is increased

Ans - D

Reason - ( DCP gets drive from the front roller via the crown wheel, so it will not have any effect on front roller rpm. DCP will give drive to the other rollers i.e. 2nd, 3rd & 4th roller. So if the DCP teeth increases the rpm of 2nd, 3rd & 4th roller will increase, since DCP is a driver. Since draft = surface speed of delivery roller divided by surface speed of feed roller, and here the surface speed of feed roller increases so the draft will decrease.)

Q.10. When the number of teeth in the twist wheel is increased

Ans - Both B & C are correct answers.

Reason - ( Twist wheel is a driver and so when the number of teeth increases the rpm of front roller will increase. Again twist = Spindle Speed divided by FRSS, so when FRSS will increase twist will decrease)