ANSWERS TO ASSIGNMENT 2

a)      Q1. Match the two columns                                                               1 X 15 = 15

| A | Top Roller | 1 | Draft |
| --- | --- | --- | --- |
| B | Roller Setting | 2 | Winding |
| C | Count | 3 | RPM reduces as bobbin builds |
| D | Twist | 4 | Roller Cots |
| E | Bobbin leading | 5 | Movement reduces as bobbin builds |
| F | DCP | 6 | Turns per unit length |
| G | Draft between 2nd & 3rd roller | 7 | Constant RPM |
| H | Bottom cone drum | 8 | Effective length |
| I | Pressure arm | 9 | Production |
| J | Bobbin Rail | 10 | 7000 grains |
| K | Spindle | 11 | Flyer |
| L | Tex | 12 | Tension draft |
| M | Front roller surface speed | 13 | Drafting wave |
| N | Bent bottom roller | 14 | 1000 metres |
| O | Pound | 15 | Change Place |

( answer should be written as A- Number which matches, so if A matches 1, then write A-1)

ANSWER:-

A-4,B-8,C-1,D-6,E-2,F-15,G-12,H-3,I-11,J-5,K-7,L-14,M-9,N-13,O-10.

Q.2 What are the “Shirley Modifications” in drafting system of S/F?                               2

ANSWER-

1. The draft distribution in a 4 over 4 drafting system was  A,B & C, in the three drafting zones, Where A is the draft in the front zone, B in the middle zone and C in the back zone. So the draft in the front zone was maximum and draft in the back zone was minimum, A>B>C. Shirley Institute suggested that this draft distribution should be changed so that the draft in the middle zone was minimum, i.e. tension draft.

2. To  change the design of the top roller flutes in such a way so as make the flutes pointed, which resulted in better penetration and better grip of fibres.

Q.3 State the functions of a flyer in a S/F?                                                                            2

ANSWER

1 It helps in twisting the fibres.

2  Assist in winding the rove on the surface of the bobbin.

3 Imparts tension on the rove resulting in a tight package.

4 Guides the rove on to the bobbin

Q.4 What is the function of apron in a drafting system                                                         1

All of you have got this right

Q.5 Enlist the functions of a building mechanism in S/F.                                                      3

All of you have got this right

Q.6 How winding is regulated in a S/F? Explain                                                                   3

ANSWER:

Winding takes place due to the difference of speed between the bobbin and flyer. This difference in speed is known as the winding rate. As the rove is being wound on the bobbin, the bobbin dia increases resulting in increase of bobbin surface speed. If bobbin surface speed is allowed to increase, then, the winding rate will also increase( in a bobbin leading machine), this cannot be to happen and to regulate the winding, the bobbin rpm is reduced as the rove is being wound on the bobbin in such a manner that the surface speed of the bobbin is always the same. We know surface speed= 𝛑*D*N, where pi is a constant and D is the diameter of the bobbin and N is the rpm. For eg, 1*12= 12, 2*6=12, 3*4=12, 4*3=12, 6*2=12, 12*1=12. To regulate the winding we have to bring down the rpm of the bobbin as the rove is being wound on the bobbin. This is done with the help building mechanism, cold drum and differential gear.

Q.7 If the draft in three zones of a drafting zone is 2, 1.2 & 4. Calculate the total draft.     1

All of you have got this right

Q.8 Explain why the winding rate must be equal to the delivery rate in S/F.                    3

ANSWER:

The delivery rate is the length of rove delivered in unit time (generally this unit time is 1 minute). Winding rate is the length of rove wound on the surface of the bobbin in unit time (generally this unit time is 1 minute). Suppose the delivery rate is 100 meters per minute, then, we will have to wind 100 meters on the bobbin every minute. If the winding rate is more than the delivery rate then the rove will be  stretched and in due course of time it will break. Whereas, if we wind less than 100 meters per minute on the bobbin then the rove between the front roller and the flyer top will become loose and hang down. When the flyers are moving an air current is generated and this air current will swing the rove which is hanging down and the rove will get entangled with the nearby roves or  flyers resulting in a breakage. So it can be seen that if the winding rate is not equal to delivery rate it will lead to breakage in the rove resulting in discontinuation of the winding process.