SPEED FRAME ASSIGNMENT 2

- Q1. Match the two columns						1 X 15 = 15

| A | Top Roller | 1 | Draft |
| --- | --- | --- | --- |
| B | Roller Setting | 2 | Winding |
| C | Count | 3 | RPM reduces as bobbin builds |
| D | Twist | 4 | Roller Cots |
| E | Bobbin leading | 5 | Movement reduces as bobbin builds |
| F | DCP | 6 | Turns per unit length |
| G | Draft between 2nd & 3rd roller | 7 | Constant RPM |
| H | Bottom cone drum | 8 | Effective length |
| I | Pressure arm | 9 | Production |
| J | Bobbin Rail | 10 | 7000 grains |
| K | Spindle | 11 | Flyer |
| L | Tex | 12 | Tension draft |
| M | Front roller surface speed | 13 | Drafting wave |
| N | Bent bottom roller | 14 | 1000 metres |
| O | Pound | 15 | Change Place |

( answer should be written as A- Number which matches, so if A matches 1, then write A-1)

Q.2 What are the “Shirley Modifications” in drafting system of S/F?      	       		2

Q.3 State the functions of a flyer in a S/F?	                                    			2

Q.4 What is the function of apron in a drafting system					1

Q.5 Enlist the functions of a building mechanism in S/F.					3

Q.6 How winding is regulated in a S/F? Explain						3

Q.7 If the draft in three zones of a drafting zone is 2, 1.2 & 4. Calculate the total draft.	1

Q.8 Explain why the winding rate must be equal to the delivery rate in S/F.       		3