SPEED FRAME QUESTIONS AND ANSWERS

1. What is the need for roving frame in the ring spinning system?

Ans: In ring spinning system, conversion of sliver into yarn through a single step has not succeeded since the total draft needed is in the range 300-500 which is very difficult to apply on sliver in a single step and obtain good quality yarn. Sliver cans occupy large space in comparison to the space of one spinning position of ring spinning frame, so there is need to have finer strand wound on smaller packages, which the roving operation satisfy. Sliver is a thick, untwisted strand that will lead to more hairs and fly while converting it directly to yarn when high draft is applied. In contrary, rovings are finer and twisted so the chances for generation of hairs and fly are less with roving operation.

2. What are the objectives of the roving operation?

Ans:  Attenuation of the sliver

Protective twist insertion.

Winding the roving on a suitable package.

Conversion of sliver to roving

3. What is the difference between the flyer lead and bobbin lead method of roving winding?

Ans: In flyer Lead, flyer surface speed is faster and flyer winds the roving on the bobbins surface In Bobbin Lead, bobbins surface speed is faster and bobbin winds the roving onto itself

4. Why bobbin lead method of roving winding is preferable than the flyer lead method for cotton?

Ans: With bobbin lead, in case of roving break, the direction of roving on the bobbins provides stable outer layer. The drive to the spindle is shortest hence it starts faster than the bobbins. This leads to more roving breaks in flyer lead while staring. For cotton system, because of the advantages of bobbin lead method and the difficulties associated with flyer lead method, the bobbin lead method is always used

.5.What are the functions of the builder motion in a roving machine?

Ans:

The rotational rate of the bobbin should be reduced for layer

formation

Shorten the lift after each layer to form tapered ends on the bobbin

Reverse the direction of movement of the bobbin rail after each

layer formation

The speed of the movement of the bobbin rail should be reduced

after formation of every layer, as it will take more time to lay one coil

as the bobbin builds up.

6. What is the roll of spacers in apron drafting system?

Ans: Pressure is applied by top aprons on the lower aprons and the distance between them decides the intensity of fibre clamping and fibre guidance. The apron arrangement must permit precise adaptation of the minimum distance to the fibre volume. It is done by placing “spacers” between the nose bar of the lower apron and the cradle edge of the top apron, i.e at the exit opening.

7. What is the function of pressure arm in roving frame flyer?

Ans:  The pressure arm made up of steel yoke is attached to the lower end of the hollow flyer leg. It guides the roving from exit of the flyer leg to the package. The roving is wrapped two or three times around the yoke. No. of turns determines the roving tension and package hardness. If this is high, then a hard, compact package is obtained. If it is too high, false drafts or roving breaks may happen.